GLOBALS {ignore}

//

// This routine computes the bounding box of a card deck, by looping through

// all cards in the deck, and grabbing the coordinates of each, while keeping

// track of the minimum and maximum values for X and Y. It uses those values

// to compute the maximum width and height of the bounding box, as well.

//

ROUTINE computeBoundingBox {} {card x1 y1 w h x2 y2 bx1 by1 bx2 by2 bw bh}

// initialize computation variables:

// lower-left (minimum) X/Y values

SET bx1 9999

SET by1 9999

// upper-right (maximum) X/Y values

SET bx2 -9999

SET by2 -9999

// maximum width/height values

SET bw 0

SET bh 0

// clear results text prior to calculations

SET dbbc_co_clear 1

// loop through all cards in the specified deck

SET card 0

LOOP card < dbbc_oi_max_num_cards

// select a card

SET @dbbc_cn_text card

// get the X/Y coordinates of the deck (i.e., of the card)

SET altiaGetObjXY dbbc_oi_integer

SET x1 altiaObjX

SET y1 altiaObjY

// get the width/height of the deck (i.e., of the card)

SET altiaGetObjWH dbbc_oi_integer

SET w altiaObjW

SET h altiaObjH

// compute the upper-right limit of the card

MATH x2 x1 + w

MATH y2 y1 + h

// determine the lower-left limit of the deck

IF x1 < bx1

SET bx1 x1

END

IF y1 < by1

SET by1 y1

END

// determine the upper-right limit of the deck

IF x2 > bx2

SET bx2 x2

END

IF y2 > by2

SET by2 y2

END

MATH card card + 1

END

// finally, select card #0

SET @dbbc_cn_text 0

// determine the maximum size of the deck

MATH bw bx2 - bx1

MATH bh by2 - by1

// display the results

EXPR dbbc_co_text {[$format "(%d,%d)-(%d,%d) %d x %d" bx1 by1 bx2 by2 bw bh]}

END

//

// This code is executed when the 'Bounding Box Compute Parameter' is set to 1,

// in the Custom Properties of the 'Deck Bounding Box Calculator' group.

//

WHEN dbbc_co_compute == 1

// perform some sanity checks

IF dbbc_oi_integer <= 0

SET dbbc_co_text "Please enter Deck Object ID first!"

RETURN 0

END

IF dbbc_cn_text < "0"

SET dbbc_co_text "Please enter Deck Card Name first!"

RETURN 0

END

IF dbbc_oi_max_num_cards <= 0

SET dbbc_co_text "Maximum Number of Cards must be at least 1!"

RETURN 0

END

// once all sanity checks pass, perform the calculations

CALL computeBoundingBox {} ignore

END